FEDERAL FLIGHT AGENCY

NETWORK DESIGN PROJECT

Final Documentation Deliverables

Group Member IDs:

22101573  |  22301500  |  22301022

Base Network: 1.0.0.0/16  (derived from 3rd member ID: 22301022 → middle digits '10')

# Topology Diagram

## Network Overview

The Federal Flight Agency Network is built on base network 1.0.0.0/16, subdivided using VLSM into 11 subnets. Five main routers interconnect via Serial WAN links and a shared Ethernet backbone. The diagram below describes the full logical layout with labeled subnets, interfaces, and sub-units.

╔══════════════════════════════════════════════════════════════════════╗ ║ FEDERAL FLIGHT AGENCY – LOGICAL TOPOLOGY ║ ╚══════════════════════════════════════════════════════════════════════╝ ┌─────────────────────────────────────────────────────────────────────────────────────┐ │ HQ-ROUTER (Command Center) │ │ LAN Fa0/0 ──► 1.0.0.1/21 [Control Wing – 2000 devices] │ │ WAN Se0/0/0 ──► 1.0.24.161/30 [Serial to SIM-ROUTER – DTE] │ │ WAN Se0/0/1 ──► 1.0.24.165/30 [Serial to EMERG-ROUTER – DTE] │ │ Servers: DNS/Web (www.federalfgt.control) @ 1.0.0.2 │ │ Email (mail.hq.federal.com) @ 1.0.0.3 │ └──────────────────────┬─────────────────────┬──────────────────────────────────────── ┘ │ Se0/0/0 (Serial DTE) │ Se0/0/1 (Serial DTE) ┌────────────────┘ └──────────────────────┐ ▼ ▼ ┌────────────────────────────┐ ┌────────────────────────────────────────┐ │ SIM-ROUTER (Sim Lab) │ │ EMERG-ROUTER (Emergency Control Node) │ │ Se0/0/0 ──► 1.0.24.162/30 │ │ Se0/0/0 ──► 1.0.24.166/30 │ │ Fa0/0 ──► 1.0.8.1/21 │ │ Fa0/0 ──► 1.0.24.1/25 │ │ [Simulation Core – 1200d] │ │ [Backup Core – 80 devices] │ │ Fa0/1 ──► 1.0.24.137/29 │ │ Static routing only; no RIPv2 │ │ [Testing Sandbox – 3d] │ └────────────────────────────────────────┘ │ DHCP Server in Sim Core │ └─────────────┬──────────────┘ │ Fa0/1 (FastEthernet to Backbone) ▼ ┌──────────────────────────────────────────────────────────────┐ │ BACKBONE SWITCH (1.0.24.128/29) │ │ [AI, SIM, HANGAR connect here via Fa/Gi ports] │ └─────────────┬────────────────────────┬────────────────────── ┘ │ │ ▼ ▼ ┌─────────────────────────────┐ ┌─────────────────────────────────────┐ │ AI-ROUTER (AI Nav Unit) │ │ HANGAR-ROUTER (Maintenance Hangar) │ │ Gi0/0 ──► 1.0.24.130/29 │ │ Fa0/0 ──► 1.0.24.131/29 │ │ Fa0/0 ──► 1.0.16.1/22 │ │ Fa0/1 ──► 1.0.20.1/22 │ │ [ML Engine – 850 devices] │ │ [Diagnostics Bay – 600 devices] │ │ Vlan1 ──► 1.0.24.137/29 │ │ Fa0/2 ──► 1.0.24.153/29 │ │ [HWIC-4ESW → Sandbox] │ │ [Repair Unit – 3 devices] │ │ Fa0/1 ──► 1.0.24.145/29 │ │ DHCP provided by HANGAR-ROUTER │ │ [Data Processing – 3d] │ │ distribute-list: blocks HQ routes │ │ DHCP provided locally │ │ from AI Unit via RIPv2 │ │ Email: mail.ai.federal.com │ └─────────────────────────────────────┘ │ @ 1.0.16.2 │ └─────────────────────────────┘ FLOATING STATIC ROUTE: AI-ROUTER → 1.0.0.0/16 via Sandbox Vlan1 (AD=63) RIPv2 DOMAIN: SIM ↔ AI ↔ HANGAR ↔ BACKBONE (Emergency excluded) STATIC ROUTING: HQ (next-hop) ↔ Emergency (exit-interface)

# VLSM Calculation Tree

## Base Network & Derivation

Group ID 3: 22301022 → middle two digits = 10 → split into two octets: 1.0 → Base Network = 1.0.0.0/16 (65,536 total addresses, 65,534 usable).

## VLSM Allocation Methodology

Subnets are allocated in descending order of host requirement. For each subnet:

1. Determine required devices + 2 (network + broadcast addresses).

2. Find the smallest power of 2 ≥ that total → this gives the block size.

3. Prefix = 32 − log₂(block size).  Usable hosts = block size − 2.

## Step-by-Step VLSM Calculations

| 1. Control Wing (HQ LAN) [Required: 2000 devices] |  |
| --- | --- |
| Calculation: | Required: 2000 + 2 (net + bcast) = 2002 Next power of 2 ≥ 2002 → 2¹¹ = 2048 Prefix = 32 − 11 = /21 Usable hosts = 2048 − 2 = 2046 ✓ (≥ 2000) |
| Result: | Network: 1.0.0.0/21 Mask: 255.255.248.0 Range: 1.0.0.1 → 1.0.7.254 Broadcast: 1.0.7.255 Next available: 1.0.8.0 |

| 2. Simulation Core [Required: 1200 devices] |  |
| --- | --- |
| Calculation: | Required: 1200 + 2 = 1202 Next power of 2 ≥ 1202 → 2¹¹ = 2048 Prefix = 32 − 11 = /21 Usable hosts = 2046 ✓ (≥ 1200) |
| Result: | Network: 1.0.8.0/21 Mask: 255.255.248.0 Range: 1.0.8.1 → 1.0.15.254 Broadcast: 1.0.15.255 Next available: 1.0.16.0 |

| 3. ML Engine (AI Navigation Unit LAN) [Required: 850 devices] |  |
| --- | --- |
| Calculation: | Required: 850 + 2 = 852 Next power of 2 ≥ 852 → 2¹⁰ = 1024 Prefix = 32 − 10 = /22 Usable hosts = 1022 ✓ (≥ 850) |
| Result: | Network: 1.0.16.0/22 Mask: 255.255.252.0 Range: 1.0.16.1 → 1.0.19.254 Broadcast: 1.0.19.255 Next available: 1.0.20.0 |

| 4. Diagnostics Bay (Maintenance Hangar LAN) [Required: 600 devices] |  |
| --- | --- |
| Calculation: | Required: 600 + 2 = 602 Next power of 2 ≥ 602 → 2¹⁰ = 1024 Prefix = 32 − 10 = /22 Usable hosts = 1022 ✓ (≥ 600) |
| Result: | Network: 1.0.20.0/22 Mask: 255.255.252.0 Range: 1.0.20.1 → 1.0.23.254 Broadcast: 1.0.23.255 Next available: 1.0.24.0 |

| 5. Backup Core (Emergency Control Node LAN) [Required: 80 devices] |  |
| --- | --- |
| Calculation: | Required: 80 + 2 = 82 Next power of 2 ≥ 82 → 2⁷ = 128 Prefix = 32 − 7 = /25 Usable hosts = 126 ✓ (≥ 80) |
| Result: | Network: 1.0.24.0/25 Mask: 255.255.255.128 Range: 1.0.24.1 → 1.0.24.126 Broadcast: 1.0.24.127 Next available: 1.0.24.128 |

| 6. Backbone Switch LAN [Required: ~5 router interfaces] |  |
| --- | --- |
| Calculation: | Required: 5 + 2 = 7 Next power of 2 ≥ 7 → 2³ = 8 Prefix = 32 − 3 = /29 Usable hosts = 6 ✓ (≥ 5) |
| Result: | Network: 1.0.24.128/29 Mask: 255.255.255.248 Range: 1.0.24.129 → 1.0.24.134 Broadcast: 1.0.24.135 Next available: 1.0.24.136 |

| 7. Testing Sandbox (Sim Lab sub-unit) [Required: min 3 devices (2 PCs + 1 Printer)] |  |
| --- | --- |
| Calculation: | Required: 3 + 2 = 5 Next power of 2 ≥ 5 → 2³ = 8 Prefix = 32 − 3 = /29 Usable hosts = 6 ✓ (≥ 3) NOTE: Also serves as AI Floating Route physical path |
| Result: | Network: 1.0.24.136/29 Mask: 255.255.255.248 Range: 1.0.24.137 → 1.0.24.142 Broadcast: 1.0.24.143 Next available: 1.0.24.144 |

| 8. Data Processing Cell (AI Unit sub-unit) [Required: min 3 devices (2 PCs + 1 Printer)] |  |
| --- | --- |
| Calculation: | Required: 3 + 2 = 5 Next power of 2 ≥ 5 → 2³ = 8 Prefix = 32 − 3 = /29 Usable hosts = 6 ✓ |
| Result: | Network: 1.0.24.144/29 Mask: 255.255.255.248 Range: 1.0.24.145 → 1.0.24.150 Broadcast: 1.0.24.151 Next available: 1.0.24.152 |

| 9. Repair Unit (Hangar sub-unit) [Required: min 3 devices (2 PCs + 1 Printer)] |  |
| --- | --- |
| Calculation: | Required: 3 + 2 = 5 Next power of 2 ≥ 5 → 2³ = 8 Prefix = 32 − 3 = /29 Usable hosts = 6 ✓ |
| Result: | Network: 1.0.24.152/29 Mask: 255.255.255.248 Range: 1.0.24.153 → 1.0.24.158 Broadcast: 1.0.24.159 Next available: 1.0.24.160 |

| 10. Serial WAN: HQ ↔ Simulation Lab [Required: 2 endpoints] |  |
| --- | --- |
| Calculation: | Required: 2 + 2 = 4 Next power of 2 ≥ 4 → 2² = 4 Prefix = 32 − 2 = /30 Usable hosts = 2 ✓ (exact point-to-point) |
| Result: | Network: 1.0.24.160/30 Mask: 255.255.255.252 Range: 1.0.24.161 → 1.0.24.162 Broadcast: 1.0.24.163 Next available: 1.0.24.164 |

| 11. Serial WAN: HQ ↔ Emergency Control [Required: 2 endpoints] |  |
| --- | --- |
| Calculation: | Required: 2 + 2 = 4 2² = 4 → Prefix = /30 Usable hosts = 2 ✓ |
| Result: | Network: 1.0.24.164/30 Mask: 255.255.255.252 Range: 1.0.24.165 → 1.0.24.166 Broadcast: 1.0.24.167 Next available: 1.0.24.168 (end of VLSM allocation) |

## Floating Static Route – AD Calculation

AD = (Sum of 1st member ID digits) × (Group Size) 1st Member ID: 22101573 Digit Sum: 2+2+1+0+1+5+7+3 = 21 Group Size: 3 members AD = 21 × 3 = 63 Command: ip route 0.0.0.0 0.0.0.0 [next-hop-in-sandbox] 63 Note: AD=63 is higher than static (AD=1) but lower than RIPv2 (AD=120), so this route is only used when primary fails.

# IP Addressing Table

## Router & Server Interface Assignments

| DEVICE | INTERFACE | IP ADDRESS | SUBNET MASK | GATEWAY |
| --- | --- | --- | --- | --- |
| HQ-ROUTER | Fa0/0 (Control Wing LAN) | 1.0.0.1 | 255.255.248.0 /21 | N/A |
|  | Se0/0/0 → Sim Lab (DTE) | 1.0.24.161 | 255.255.255.252 /30 | N/A |
|  | Se0/0/1 → Emergency (DTE) | 1.0.24.165 | 255.255.255.252 /30 | N/A |
| SIM-ROUTER | Se0/0/0 → HQ | 1.0.24.162 | 255.255.255.252 /30 | N/A |
|  | Fa0/0 (Simulation Core LAN) | 1.0.8.1 | 255.255.248.0 /21 | N/A |
|  | Fa0/1 (Testing Sandbox) | 1.0.24.137 | 255.255.255.248 /29 | N/A |
|  | Fa1/0 (to Backbone Switch) | 1.0.24.132 | 255.255.255.248 /29 | N/A |
| AI-ROUTER | Gi0/0 (to Backbone Switch) | 1.0.24.130 | 255.255.255.248 /29 | N/A |
|  | Fa0/0 (ML Engine LAN) | 1.0.16.1 | 255.255.252.0 /22 | N/A |
|  | Fa0/1 (Data Processing Cell) | 1.0.24.145 | 255.255.255.248 /29 | N/A |
|  | Vlan1 HWIC-4ESW (→ Sandbox) | 1.0.24.138 | 255.255.255.248 /29 | N/A |
| HANGAR-ROUTER | Fa0/0 (to Backbone Switch) | 1.0.24.131 | 255.255.255.248 /29 | N/A |
|  | Fa0/1 (Diagnostics Bay LAN) | 1.0.20.1 | 255.255.252.0 /22 | N/A |
|  | Fa0/2 (Repair Unit) | 1.0.24.153 | 255.255.255.248 /29 | N/A |
| EMERG-ROUTER | Se0/0/0 → HQ | 1.0.24.166 | 255.255.255.252 /30 | N/A |
|  | Fa0/0 (Backup Core LAN) | 1.0.24.1 | 255.255.255.128 /25 | N/A |
| DNS/Web Server (HQ) | NIC (Control Wing LAN) | 1.0.0.2 | 255.255.248.0 /21 | 1.0.0.1 |
| Email Server HQ | NIC (Control Wing LAN) | 1.0.0.3 | 255.255.248.0 /21 | 1.0.0.1 |
| Email Server AI | NIC (ML Engine LAN) | 1.0.16.2 | 255.255.252.0 /22 | 1.0.16.1 |

# Cisco IOS CLI Configurations

## 4.1  HQ-ROUTER  (Command Center)

enable configure terminal hostname HQ-ROUTER interface FastEthernet0/0 ip address 1.0.0.1 255.255.248.0 description Control-Wing-LAN no shutdown interface Serial0/0/0 ip address 1.0.24.161 255.255.255.252 description WAN-to-SIM-DTE clock rate 64000 no shutdown interface Serial0/0/1 ip address 1.0.24.165 255.255.255.252 description WAN-to-EMERGENCY-DTE clock rate 64000 no shutdown ip route 1.0.24.0 255.255.255.128 1.0.24.166 ip route 1.0.8.0 255.255.248.0 1.0.24.162 ip route 1.0.24.128 255.255.255.248 1.0.24.162 ip route 1.0.24.136 255.255.255.248 1.0.24.162 ip dns server ip host www.federalfgt.control 1.0.0.2 ip host mail.hq.federal.com 1.0.0.3 ip host mail.ai.federal.com 1.0.16.2 ip name-server 1.0.0.2 end write memory

## 4.2  SIM-ROUTER  (Aircraft Simulation Lab)

enable configure terminal hostname SIM-ROUTER interface Serial0/0/0 ip address 1.0.24.162 255.255.255.252 description WAN-to-HQ no shutdown interface FastEthernet0/0 ip address 1.0.8.1 255.255.248.0 description Sim-Core-LAN no shutdown interface FastEthernet0/1 ip address 1.0.24.137 255.255.255.248 description Testing-Sandbox no shutdown interface FastEthernet1/0 ip address 1.0.24.132 255.255.255.248 description Backbone-Switch-Uplink no shutdown ip route 1.0.0.0 255.255.248.0 1.0.24.161 ip route 1.0.24.164 255.255.255.252 1.0.24.161 router rip version 2 no auto-summary network 1.0.8.0 network 1.0.24.128 network 1.0.24.136 network 1.0.24.132 passive-interface Serial0/0/0 ip dhcp pool SIM-CORE network 1.0.8.0 255.255.248.0 default-router 1.0.8.1 dns-server 1.0.0.2 lease 7 ip dhcp excluded-address 1.0.8.1 1.0.8.10 end write memory

## 4.3  AI-ROUTER  (AI Navigation Unit)

enable configure terminal hostname AI-ROUTER interface GigabitEthernet0/0 ip address 1.0.24.130 255.255.255.248 description Backbone-Switch-Uplink no shutdown interface FastEthernet0/0 ip address 1.0.16.1 255.255.252.0 description ML-Engine-LAN no shutdown interface FastEthernet0/1 ip address 1.0.24.145 255.255.255.248 description Data-Processing-Cell no shutdown interface Vlan1 ip address 1.0.24.138 255.255.255.248 description HWIC-Vlan1-to-Sandbox no shutdown router rip version 2 no auto-summary network 1.0.24.128 network 1.0.16.0 network 1.0.24.144 passive-interface FastEthernet0/0 passive-interface FastEthernet0/1 ip route 1.0.0.0 255.255.248.0 1.0.24.137 63 ip dhcp pool ML-ENGINE network 1.0.16.0 255.255.252.0 default-router 1.0.16.1 dns-server 1.0.0.2 lease 7 ip dhcp excluded-address 1.0.16.1 1.0.16.10 ip dhcp excluded-address 1.0.16.2 ip dhcp pool DATA-PROC network 1.0.24.144 255.255.255.248 default-router 1.0.24.145 dns-server 1.0.0.2 ip dhcp excluded-address 1.0.24.145 end write memory

## 4.4  HANGAR-ROUTER  (Maintenance Hangar)

enable configure terminal hostname HANGAR-ROUTER interface FastEthernet0/0 ip address 1.0.24.131 255.255.255.248 description Backbone-Switch-Uplink no shutdown interface FastEthernet0/1 ip address 1.0.20.1 255.255.252.0 description Diagnostics-Bay-LAN no shutdown interface FastEthernet0/2 ip address 1.0.24.153 255.255.255.248 description Repair-Unit no shutdown access-list 10 deny 1.0.0.0 0.0.7.255 access-list 10 permit any router rip version 2 no auto-summary network 1.0.24.128 network 1.0.20.0 network 1.0.24.152 distribute-list 10 in FastEthernet0/0 passive-interface FastEthernet0/1 passive-interface FastEthernet0/2 ip dhcp pool DIAG-BAY network 1.0.20.0 255.255.252.0 default-router 1.0.20.1 dns-server 1.0.0.2 lease 7 ip dhcp excluded-address 1.0.20.1 1.0.20.10 ip dhcp pool REPAIR-UNIT network 1.0.24.152 255.255.255.248 default-router 1.0.24.153 dns-server 1.0.0.2 ip dhcp excluded-address 1.0.24.153 end write memory

## 4.5  EMERG-ROUTER  (Emergency Control Node)

enable configure terminal hostname EMERG-ROUTER interface Serial0/0/0 ip address 1.0.24.166 255.255.255.252 description WAN-to-HQ no shutdown interface FastEthernet0/0 ip address 1.0.24.1 255.255.255.128 description Backup-Core-LAN no shutdown ip route 1.0.0.0 255.255.248.0 Serial0/0/0 ip route 1.0.24.160 255.255.255.252 Serial0/0/0 ip route 1.0.8.0 255.255.248.0 Serial0/0/0 ip route 1.0.16.0 255.255.252.0 Serial0/0/0 ip route 1.0.20.0 255.255.252.0 Serial0/0/0 ip route 1.0.24.128 255.255.255.248 Serial0/0/0 ip route 1.0.24.136 255.255.255.248 Serial0/0/0 ip route 1.0.24.144 255.255.255.248 Serial0/0/0 ip route 1.0.24.152 255.255.255.248 Serial0/0/0 end write memory

## 4.6  Implementation Notes & Verification Commands

| Verification Command | Purpose |
| --- | --- |
| show ip route | Verify routing table on each router |
| show ip rip database | Check RIPv2 learned routes (SIM/AI/Hangar) |
| show ip interface brief | Confirm all interfaces are up/up |
| show ip dhcp binding | Verify DHCP leases are assigned |
| debug ip rip | Troubleshoot RIPv2 updates |
| show access-lists | Confirm ACL 10 on Hangar distribute-list |
| ping 1.0.0.2 | Test reachability to DNS/Web server at HQ |
| show ip static route | Verify floating static AD=63 on AI-ROUTER |

IMPORTANT: The HWIC-4ESW module on AI-ROUTER must be physically cabled from one of its switch ports to the same switch as the Testing Sandbox. The Vlan1 interface on the HWIC module bridges into that subnet, providing the physical path for the floating static route.

RIPv2 Domain Summary: SIM-ROUTER, AI-ROUTER, and HANGAR-ROUTER form the RIPv2 domain. HQ-ROUTER and EMERG-ROUTER are excluded — HQ uses static routing only; Emergency uses exit-interface static routing only.